Smoke Testing Is Coming To XXXX Township!

“I&I” is a short acronym for a huge problem most sewered communities face, Infiltration and Inflow.  Inflow occurs when rainwater is misdirected into the sanitary sewer system instead of storm sewers.  Examples are: roof leaders, yard and area drains, manhole covers, and cross connections from storm drains. The remedy for inflow is to remove improper connections to the sanitary sewer system.  Infiltration occurs when ground water seeps into the sanitary sewer system through cracks or leaks in sewer pipes. The cracks or leaks may be caused by age related deterioration, loose joints, damage or root infiltration. The remedy for infiltration is repairing or replacing the leaking infrastructure.  Ignored I&I results in excessive flows in the sewers and into the wastewater treatment plant.  This becomes a very costly problem for all of us.  We have more than 100 miles of sewers underground and it’s difficult to find these leaks.  Smoke testing can help us and our residents.

Smoke testing can be a very efficient and cost effective way to locate and identify the source of an inflow or infiltration problem.   The smoke used is non-toxic and does not leave any residue.  This special smoke is applied under pressure and will fill the sewer main line plus any connection, and also will follow the path of a leak to the ground surface, quickly revealing the source of I&I.  See the picture.  Only enough force to overcome atmospheric pressure is required.  The crews will be checking the area for an escaping smoke.  They will carefully check around houses with close attention given to cleanouts and roof leaders.  We may see smoke coming out of the grass, wooded areas, or cracks in the pavement, indicating leaks.  If smoke is found during the inspection, it will be carefully recorded on camera so that problems can then be corrected.  Best results are obtained when the water tables are low and on dry days since water is an excellent smoke barrier. Smoke testing is avoided on windy days because even a very light breeze can disperse a wisp of smoke before it is visible at the source of a leak.

Letters will be mailed and notices will be posted notifying residents of when the smoke testing will occur.  Only the properties that are in the surrounding area of the vicinity that has been chosen to test will be notified.   A notification letter will be sent approximately two weeks in advance of the starting test date that includes all pertinent information to the homeowner or resident.

The notices will include the following information along with the testing date:

The non-toxic smoke is manufactured specifically for this purpose, leaves no residuals or stains, and has no effects on plants or animals. It has a distinctive, but not unpleasant, odor. The visible smoke and odor last only a few minutes where there is adequate ventilation.

The smoke should not enter your home.  However, if smoke does enter your home any of the following items are the probable cause.

- The vents connected to your building’s sewer lateral are inadequate, defective, or improperly installed.

- The traps under sinks, tubs, basins, showers, floor drains, etc. are dry, defective, improperly installed, or missing.

- The pipes, connections, and seals of the building’s sewer system are damaged, defective, have plugs missing, or are improperly installed.

During the week prior to test date, pour water down ALL drains in your home or building to make sure that all your drain traps are full.

If traces of the smoke or its odor enter your house or building, it is an indication that odor from the sewer system may also be entering.  Sewer gases can be unpleasant, dangerous, and a potential health hazard.  The location, identification, and correction of the source of any smoke entering your house is urgently advised.

The Township can provide assistance in locating the source of smoke entering your house. However, correction of any defects in the pipes and sewer on private property is the responsibility of the owner. If smoke is observed in your home and the source is not readily identified or you have any questions, please call XXX-XXXX

Sincerely,

A news release and smoke testing CAUTION letter will be sent out to the media and other officials, including the fire departments and police, to let them know our plans.  This is usually done one week in advance.  The news release should include the days and exact locations, why we are doing the test and where they can expect to see smoke.  Our phone number for questions will included.  Remember newsletter article is just a general forewarning.  Your notification letter will cover all of the necessary details if you happen to live in the testing area.  Door hangers will also be distributed in the testing areas.

The smoke testing area will probably be the area XXXXXXXXXXXXXX.  Testing will be done this summer, on dry and still days.